CONCRETE ESTIMATE

| CONCRETE CONTRACTOR Business name: Example Concrete Co. Address: 7 Example Yard Road, Sampletown Phone / email: [Phone or email] License no.: [License no., if required where you work] | ESTIMATE DETAILS Estimate no.: CN-1185 Date: Nov 5, 2026 Valid until: Dec 5, 2026 Prepared by: [Estimator name] |
| --- | --- |

CLIENT AND JOB SITE

| Client name | Alex Example | Phone / email | [Phone or email] |
| --- | --- | --- | --- |
| Job address | 92 Sample Avenue, Sampletown |  |  |
| Existing surface | Cracked asphalt driveway, about 4 in. thick |  |  |

SLAB DETAILS

| Slab type | Driveway | Area | 640 sq ft (16 ft by 40 ft) |
| --- | --- | --- | --- |
| Thickness | 4 in. |  |  |
| Concrete volume | 7.9 cu yd, plus about 7% for waste and grade = 8.5 cu yd |  |  |
| Finish | Broom finish, control joints about every 10 ft |  |  |

REMOVAL AND BASE

| Description | Qty | Unit | Unit price | Amount |
| --- | --- | --- | --- | --- |
| Break out the old asphalt driveway and load for hauling | 640 | sq ft | 2.25 | 1,440.00 |
| Compacted gravel base, 4 in. | 640 | sq ft | 1.60 | 1,024.00 |
| Removal and base subtotal |  |  |  | 2,464.00 |

FORMING AND REINFORCEMENT

| Description | Qty | Unit | Unit price | Amount |
| --- | --- | --- | --- | --- |
| Forms along the edges | 112 | lin ft | 4.50 | 504.00 |
| Rebar, #4 at 18 in. on center, on chairs | 640 | sq ft | 1.35 | 864.00 |
| Expansion joint at the garage slab | 16 | lin ft | 3.00 | 48.00 |
| Forming and reinforcement subtotal |  |  |  | 1,416.00 |

CONCRETE AND FINISHING

| Description | Qty | Unit | Unit price | Amount |
| --- | --- | --- | --- | --- |
| Ready-mix concrete, 4,000 psi | 8.5 | cu yd | 165.00 | 1,402.50 |
| Place and finish, broom finish | 640 | sq ft | 2.40 | 1,536.00 |
| Saw-cut control joints | 128 | lin ft | 1.25 | 160.00 |
| Cure and seal | 640 | sq ft | 0.45 | 288.00 |
| Concrete and finishing subtotal |  |  |  | 3,386.50 |

| Subtotal | 7,266.50 |
| --- | --- |
| Disposal fee, passed through | 380.00 |
| Sales tax on work and materials, if applicable (0%) | 0.00 |
| Estimated total | 7,646.50 |

SCOPE NOTES

- Concrete volume: 640 sq ft at 4 in. thick is 7.9 cu yd; 8.5 cu yd is ordered to allow for waste and grade

- The disposal fee is the charge for hauling and dumping the old asphalt, passed through at cost

- Vehicles stay off the new driveway for 7 days after the pour

EXCLUSIONS

- Permit fees, if any

- Colored, stamped or exposed-aggregate finishes

- Removing tree roots larger than 2 in.

- Repairing landscaping or sprinkler lines disturbed at the edges

| Accepted by (client signature) |  | Print name |
| --- | --- | --- |
| Date |  |  |

Prices are based on the area and thickness shown above and are valid until the date shown.